# Known Issues

## 1. STM32 firmware cannot be reliably updated from the SD card (use ST-LINK)

### Summary

| What works | ESP32 self-update from SD ✅ · STM32 update via ST-LINK ✅ |
| --- | --- |
| What doesn’t | STM32 update from the SD card over UART ❌ (unreliable) |
| Root cause | No free ESP32 GPIO is broken out to drive the STM32 BOOT0 pin, and the STM32G4 ROM bootloader is not USART-responsive after a software-only jump |
| Workaround | Flash the STM32 with an ST-LINK programmer |
| Future fix | Move the display tier to an ESP32-P4 (more usable GPIO) and add a BOOT0 control wire, or add a tiny GPIO expander / spare pin for BOOT0 |

### The feature, and what it’s supposed to do

The product goal is a fully self-contained field update: drop a new /stm32_firmware.bin on the SD card, tap “Update STM32” on the touchscreen, and the ESP32 reflashes the STM32 over the existing UART link — no programmer, no PC. The ESP32 already does exactly this for its own firmware.

To flash the STM32 over UART, the chip must first be in its factory ROM bootloader (the ST AN3155 protocol). On a normal STM32 design you enter the ROM bootloader by asserting the BOOT0 pin high and resetting — the chip then starts the ROM bootloader instead of the application, and the bootloader listens on USART1 (PA9/PA10) at 115200 8E1.

That is where this hardware falls short.

### Why it happens (the detailed root cause)

#### a) There is no BOOT0 control wire — and nowhere to add one

The STM32’s BOOT0 is on PB8 and is not wired to the ESP32. To drive it in software we’d need a spare ESP32 GPIO routed to PB8. The Sunton ESP32-8048S043C has essentially no free, broken-out GPIO:

| GPIO(s) | Consumed by |
| --- | --- |
| 1–9, 14–16, 21, 39–42, 45–48 | RGB LCD parallel bus (16 data + DE/VSYNC/HSYNC/PCLK) |
| 19, 20 | GT911 capacitive touch I²C (SDA / SCL) |
| 17, 18 | STM32 UART link (RX / TX) |
| 10–13 | microSD card SPI (CS / MOSI / SCK / MISO) |
| 43, 44 | USB-serial debug console (UART0) |
| 0 | BOOT button |
| 26–37 | SPI flash + PSRAM (octal) |

Only pins 11/12/13, 17/18, 19/20, 43/44 reach a physical header (P1–P4), and every one of them is already in use — SD card, STM32 UART, touch, or debug console. The internally-free GPIOs (e.g. GPIO38) are not bonded to any accessible pad. So there is no pin to wire to BOOT0 without sacrificing the touchscreen, the SD card, or the debug console.

#### b) The software-only jump does not wake the G4 ROM bootloader

Because we can’t touch BOOT0, we tried entering the bootloader purely in software: the running app jumps to system memory (0x1FFF0000) via VTOR / __set_MSP / a memory remap, mimicking a BOOT0=1 reset.

On-hardware testing proved this does not work on the STM32G4: - The STM32 received the BOOTLOADER command, ACK’d it, and executed the jump (confirmed in the serial logs). - But the ROM bootloader then never responded to the AN3155 0x7F auto-baud byte — no 0x79 ACK, on every one of 10 sync attempts.

This is a known STM32G4 behavior: the ROM bootloader expects a true reset with BOOT0 asserted, not a warm jump from a running application. After a software jump the USART interface frequently comes up dead on G4/L4/H7 (unlike F4, where the same trick usually works). The application’s jump sequence is textbook — this is a silicon / ROM limitation, not a bug in our code.

#### Things that were verified along the way (so they’re ruled out)

- UART pins match — the app and the ROM bootloader both use USART1 PA9/PA10; no remap mismatch.

- Parity — the ESP32 side correctly sends 8E1 (ESP-IDF UART_PARITY_EVEN); TX bytes confirmed leaving GPIO18.

- Wiring — the same two wires carry a working 2 Mbaud app link in both directions, so 115200 is trivially within their capability.

- Command parsing — the STM32 parser matches BOOTLOADER exactly (strcmp == 0) and ACKs.

- Watchdog — the IWDG was disabled so it can’t reset the chip out of the bootloader.

Every layer except “a software jump produces a responsive ROM bootloader” checks out. That one layer is the wall.

### Workaround: flash the STM32 with an ST-LINK

This is the supported, reliable path and works every time.

- Connect an ST-LINK V2/V3 to the STM32 SWD header (SWDIO, SWCLK, GND, 3V3 reference).

- Build the firmware:

- cd STM32G474CE
pio run -e g474ceu6_stlink

- Flash it:

- pio run -e g474ceu6_stlink -t upload

- …or load STM32G474CE/.pio/build/g474ceu6_stlink/firmware.bin at address 0x08000000 with STM32CubeProgrammer.

The ESP32 continues to self-update from the SD card normally — only the STM32 needs the programmer.

#### Experimental (unverified) option-byte SD-flash path

The firmware also contains an experimental wire-free attempt: on the BOOTLOADER command the STM32 reprograms its boot option bytes (nSWBOOT0=0, nBOOT0=1, nBOOT1=1) and launches them, which performs a real reset into the ROM bootloader (electrically like BOOT0=1). The freshly flashed app then restores the option bytes (nSWBOOT0=1) on its first boot.

- Status: UNVERIFIED on hardware. It compiles and is logically sound, but has not been confirmed to complete a full flash cycle.

- Risk: if the post-flash “Go”/launch ever fails, the option bytes stay in “force bootloader” mode and the chip will re-enter the bootloader on every reset until reflashed.

- Always keep an ST-LINK handy — it is the guaranteed recovery path and will reset the option bytes back to normal.

### Future solution

- ESP32-P4 display board (preferred). The ESP32-P4 has substantially more usable GPIO. Moving Tier 2 to a P4-based panel frees pins to run a dedicated BOOT0 (and ideally NRST) control wire to the STM32. That enables the ST-supported hardware bootloader entry (assert BOOT0, pulse reset), which is rock-solid — and makes true wire-free SD updates of the STM32 possible.

- Spare pin / GPIO expander on a board respin. If staying on an ESP32-S3 class panel, route one spare GPIO (or an I²C GPIO expander output) to STM32 BOOT0 during the next PCB revision.

- Validate the option-byte path. With careful on-hardware testing and a guaranteed ST-LINK fallback, the experimental option-byte method could be promoted to the supported SD-flash path without any hardware change.

Until one of the above lands, use an ST-LINK to update the STM32.