# AGENTS.md — Spot-Welder multi-firmware workspace

Guidance for AI coding agents working in this repo. Read this before editing.

## Repository map

This is a mono-repo containing multiple independent firmware/software projects that communicate over a shared serial/TCP telemetry protocol.

| Path | Target | Toolchain | Role |
| --- | --- | --- | --- |
| ESP32P4/ | ESP32-P4 (CrowPanel Advance 5”) | ESP-IDF (CMake) | UI + WiFi bridge + STM32/OTA flasher. Talks to STM32 over UART3-IN @ 576000. Enriches STATUS, relays WAVEFORM_* raw. |
| STM32G474CE/ | STM32G474CE | PlatformIO (env:g474ceu6_stlink) | Real-time weld controller: FET drive, ADC/shunt, INA226, charger, joule integrator. Sole producer of STATUS, WELD_DONE, WAVEFORM_* packets. |
| ESP32_8048S043C/ | ESP32-8048S043C (old board) | PlatformIO / Arduino | Legacy display firmware. Produces DISPLAY and CELLS packets. Keep buildable; not the primary target. |
| (separate repo) Spot-Welder-Server/ | Python 3 / Flask | pip | Dashboard + TCP client to the P4 bridge (:8888). Re-parses telemetry. |

The Flask server lives in a separate repository (Spot-Welder-Server). For any change that touches the wire protocol, that repo MUST be edited in the same coordinated change.

## ⚠️ Source of truth: the telemetry protocol

There is currently no shared protocol header. The STATUS / WAVEFORM_DATA packet is:

- Emitted in STM32G474CE/src/main.c (search "STATUS,") as an ASCII key=value CSV line.

- Parsed in ESP32P4/main/wifi_bridge.cpp and welder_main.cpp (sscanf/strtok).

- Parsed again in Spot-Welder-Server/app.py.

See PROTOCOL.md at the repo root for a descriptive reference of observed packet formats.

Rule: any change to a field name, order, type, or units is a breaking change across all three. When editing the protocol, update ALL of: the STM32 emitter, both P4 parsers, and the Flask parser — and keep this list in sync. Prefer additive changes (append new fields) over reordering.

## Build & validation commands

Always build the project(s) you touched before claiming a change is done.

### ESP32-P4 (ESP-IDF)

cd ESP32P4
idf.py set-target esp32p4      # first time only
idf.py build                   # compile — MUST pass with no errors
idf.py -p <PORT> flash monitor # flash + serial (hardware only)

- Output binary: build/esp32_firmware.bin (project name is esp32_firmware).

- Do NOT flash hello_world.bin — that name is stale/removed.

### STM32G474CE (PlatformIO)

cd STM32G474CE
pio run                              # compile env:g474ceu6_stlink — MUST pass
pio run -t upload                    # flash via ST-Link (hardware only)
pio device monitor -b 576000         # serial monitor at app baud

### Legacy ESP32-8048S043C (PlatformIO)

cd ESP32_8048S043C
pio run                              # keep it compiling; secondary priority

### Flask server (separate repo)

cd Spot-Welder-Server
pip install -r requirements.txt
python app.py                        # serves dashboard on :8080, TCP bridge client to P4 :8888

## Hardware / safety constraints the agent MUST respect

- UART link is 576000 8N1 through BSS138 level shifters on the UART3-IN header (GPIO 27/28 on P4, PA9/PA10 on STM32). Do NOT raise the baud rate — higher rates corrupt through the RC-limited shifters. Bootloader mode uses 115200 8E1.

- WiFi is an external XIAO ESP32-C6 on SPI (J7 header), not the board’s onboard C6. The onboard C6 uses SDIO, which collides with the SD card’s SDMMC controller; the external C6-over-SPI workaround frees the SD card. Pins: MOSI 48 / MISO 47 / CLK 26 / CS 29 / HS 30 / DR 31 / reset 32 (active-low), SPI mode 3. Do NOT raise CONFIG_ESP_HOSTED_SPI_FREQ_ESP32C6 above 10 MHz (GPIO-matrix slave sampling limit). The C6 transport lives in ESP32P4/sdkconfig.defaults; full writeup in docs/hardware/ESP32C6_WIFI_DAUGHTERBOARD.md.

- The charge MOSFET (CHARGER_EN) is owned by the STM32 and MUST be OFF during a weld pulse — the joule integrator assumes discharge-only current. Never move this control to the P4.

- Reserved: do not bind host ports 1000/2200 in any tooling.

## Conventions

- C/C++: match existing style in each project (no reformatting unrelated code).

- Guard all float telemetry math with isfinite() + a < 0 floor, as the STM32 code already does.

- Never commit, push, open a PR, merge, or auto-merge unless I explicitly request it. When changes are complete, provide a per-repository diff summary.

- After any protocol edit, state explicitly which of the 3 parsers you updated.