# ESP32-P4 SPI WiFi Integration Checklist

## ✅ What’s Been Done

### Stage 1: XIAO C6 Slave Firmware (Complete)

- ✅ Seeed XIAO ESP32-C6 flashed with esp-hosted SPI slave firmware v2.12.9

- ✅ SPI transport initialized: MOSI=18, MISO=20, CLK=19, CS=2, HS=1, DR=17

- ✅ XIAO boots successfully and waits for P4 host

### Stage 2: P4 Host Firmware Reconfigured (Complete — This Commit)

- ✅ sdkconfig.defaults switched from SDIO → SPI transport

- ✅ SPI master GPIOs mapped to J7 wireless module header

- ✅ Pin configuration matches XIAO slave exactly

- ✅ Committed to Dev branch (b399380)

## 🔧 Next Steps: Hardware + Testing

### Step 1: Physical Wiring (J7 Header → XIAO C6)

CRITICAL: Wire the XIAO C6 to the CrowPanel J7 wireless module header:

| Signal | P4 GPIO | J7 Pin | XIAO Pin | XIAO GPIO |
| --- | --- | --- | --- | --- |
| MOSI | 48 | — | D10 | 18 |
| MISO | 47 | — | D9 | 20 |
| CLK | 26 | — | D8 | 19 |
| CS | 29 | — | D2 | 2 |
| Handshake | 30 | — | D1 | 1 |
| Data Ready | 31 | — | D7 | 17 |
| Power | — | 3V3 | 3V3 | — |
| Ground | — | GND | GND | — |

⚠️ DO NOT connect XIAO D4/D5 (GPIO22/23) — they’re physically wired to the P4’s I2C bus (GT911 touch + STC8 backlight controller).

Verify wiring: - Use short jumper wires (< 10 cm) to minimize signal noise - Check continuity with a multimeter if available - Ensure XIAO is powered (3.3V on J7 header)

### Step 2: Rebuild & Flash P4 Firmware

The P4 firmware MUST be rebuilt with the new SPI config:

# On your Windows machine (ESP-IDF PowerShell):
cd C:\path\to\Spot-Welder\ESP32P4
git pull origin Dev  # Get the latest SPI config

# Clean old SDIO config artifacts
idf.py fullclean

# Rebuild with new SPI config
idf.py build

# Flash to P4
idf.py -p COM6 flash monitor

### Step 3: Verify Boot Sequence

Watch the serial monitor output after flashing. You should see:

#### ✅ Expected Success Output:

I (xxxx) wifi_stack_init: Initializing ESP-Hosted...
I (xxxx) transport: SPI transport initialized
I (xxxx) transport: MOSI[48] MISO[47] CLK[26] CS[29] HS[30] DR[31]
I (xxxx) transport: Waiting for slave...
I (xxxx) transport: Slave ready, ESP-Hosted link UP
I (xxxx) wifi_bridge: WiFi stack initialized successfully

#### ❌ If You See Errors:

“Slave not responding” / “ESP-Hosted link timeout”: - Check XIAO is powered and running (use a USB cable to XIAO to verify it boots) - Check XIAO serial monitor shows “SPI slave initialized, waiting for host…” - Verify all 8 signal wires (6 SPI + 3V3 + GND) are connected correctly - Check for loose connections on J7 header

“SPI transaction failed” / “Checksum error”: - SPI clock may be too fast (20 MHz default) - Lower it in sdkconfig.defaults: CONFIG_ESP_HOSTED_SPI_FREQ_ESP32C6=10 - Rebuild and reflash P4

“SDIO timeout” (old message): - The build used the old SDIO config - Run idf.py fullclean to wipe stale sdkconfig - Rebuild from scratch

### Step 4: Test WiFi Functionality

Once the P4 boots with “ESP-Hosted link UP”:

- WiFi provisioning: Open the welder UI, navigate to WiFi settings

- Connect to your network: Enter SSID + password

- Verify connection: Check the WiFi status icon on the main screen

- Test OTA: Go to Settings → OTA Update, verify the firmware update flow works

## 🔍 Troubleshooting

### P4 boots but “WiFi disabled” (link failed)

- This is expected if XIAO isn’t connected or running

- The welder UI will work normally (WiFi just disabled)

- Fix: Wire the XIAO, verify it’s flashed and running, reboot P4

### WiFi connects but SD card doesn’t work

- ✅ This is exactly what we fixed! The SDMMC controller is now free.

- Test the SD card separately (file browser, data logging, etc.)

- If SD card still fails, check the SD card slot wiring (separate issue)

### XIAO slave stops responding after P4 reboot

- The XIAO might need a power cycle when P4 reboots

- Add a reset pulse: Connect XIAO EN pin to a P4 GPIO and toggle it in wifi_stack_init()

- For now: manually power-cycle the XIAO after P4 reboots

## 📊 Performance Tuning (Optional)

Once WiFi is stable, you can increase SPI clock speed for better throughput:

In ESP32P4/sdkconfig.defaults, change:

CONFIG_ESP_HOSTED_SPI_FREQ_ESP32C6=20  # Start here (conservative)
# Try increasing to 30, then 40 (max for C6)

Rebuild, flash, and test WiFi stability after each increase.

## 🎯 Success Criteria

You’re done when: - ✅ P4 boots with “ESP-Hosted link UP” - ✅ WiFi connects to your network - ✅ SD card mounts successfully - ✅ Welder UI fully functional with WiFi + SD card both working

## 📝 Notes

- Stable baseline preserved: Your current working firmware is now in main (commit 567d197)

- Dev branch is safe to experiment: Any issues, just revert to main

- XIAO firmware unchanged: If you need to reflash XIAO, use /home/ubuntu/xiao-c6-slave.tar.gz

Need help? Paste the P4 serial monitor output showing the WiFi init sequence.