# PROTOCOL.md — Spot Welder Serial/TCP Telemetry Protocol

Reverse-engineered descriptive reference — documents observed behavior as of the current firmware.

This is not a shared header file. Protocol definitions are independently implemented in: - STM32G474CE/src/main.c (packet producers) - ESP32P4/main/welder_main.cpp and wifi_bridge.cpp (relay + enrichment) - ESP32_8048S043C/src/main.cpp (legacy firmware) - Spot-Welder-Server/app.py (Flask consumer)

Any field name, type, or unit change requires coordinated updates across all four codebases.

## Protocol Overview

Transport: ASCII lines over UART (576000 8N1) and TCP (:8888).
Format: Comma-separated PACKET_TYPE,key=value,key=value,... or positional CSV.

### Component Roles

| Component | Role |
| --- | --- |
| STM32G474CE | Sole producer of STATUS, STATUS2, WELD_DONE, and WAVEFORM_* packets. Real-time weld controller with ADC/shunt/INA226 telemetry. |
| ESP32-P4 | Enriches STATUS by appending WiFi/system/energy fields. Relays STATUS2, WAVEFORM_*, and WELD_DONE packets raw (transparent passthrough). Produces DISPLAY packets at 1 Hz for UI smoothed voltages. |
| ESP32_8048S043C (legacy) | Produces DISPLAY and CELLS packets. Relays WAVEFORM_* packets. Not the primary firmware. |
| Flask Server | Consumes all packets. Re-parses telemetry for web dashboard. |

## Packet Catalog

### STATUS — System State and Weld Parameters

Producer: STM32
Enricher: ESP32-P4 (appends WiFi/system/energy fields)
Consumer: Flask
Frequency: ~10 Hz during idle, faster during weld

STM32-produced fields:

| Field | Type | Units | Description | Notes |
| --- | --- | --- | --- | --- |
| mode | string | — | Control mode: JOULE, MANUAL, DUAL |  |
| power | uint8 | Percent | Weld power setpoint, clamped 50–100 | Not watts. Represents PWM duty cycle percentage. |
| pulse_ms | uint16 | milliseconds | Weld pulse duration |  |
| preheat_ms | uint16 | milliseconds | Preheat phase duration |  |
| gap_ms | uint16 | milliseconds | Gap between preheat and main weld |  |
| joule_target_j | float | joules | Target energy for joule mode | Primary field (see legacy note below). |
| joule_target | float | joules | Legacy/deprecated alias of joule_target_j | Emitted only in joule mode; redundant. |
| vcap | float | volts | Capacitor bank voltage |  |
| temp | float | °C | Thermistor temperature |  |
| ichg | float | amps | Charger current (from ADC shunt) |  |
| chg_en | uint8 | boolean | Charger MOSFET state (1=on, 0=off) |  |
| state | string | — | Weld state machine: IDLE, WELD, DONE, etc. |  |
| fp | uint8 | boolean | Foot pedal state (1=pressed, 0=released) |  |

ESP32-P4 enrichment fields (appended after STM32 fields):

| Field | Type | Units | Description |
| --- | --- | --- | --- |
| rssi | int8 | dBm | WiFi signal strength |
| heap | uint32 | bytes | Free heap memory |
| uptime | uint32 | seconds | Firmware uptime |
| weld_count | uint32 | — | Total weld count (persisted in NVS) |
| energy_lifetime_j | float | joules | Cumulative energy across all welds |

Legacy note: joule_target is a redundant alias of joule_target_j, both populated from the same joule_target_j variable in main.c:679-680. Flask prefers joule_target_j.

### STATUS2 — Battery Pack Telemetry

Producer: STM32 (via INA226 sensors)
Consumer: Flask
Relay: ESP32-P4 (raw passthrough, no enrichment)
Frequency: ~10 Hz

| Field | Type | Units | Description | Notes |
| --- | --- | --- | --- | --- |
| ina_ok | uint8 | boolean | INA226 sensor health (1=ok, 0=fault) |  |
| chg_en | uint8 | boolean | Charger enable state | Duplicate of STATUS.chg_en |
| vpack | float | volts | Total pack voltage (sum of cells) |  |
| vlow | float | volts | Lowest cell voltage |  |
| vmid | float | volts | Middle cell voltage |  |
| cell1 | float | volts | Cell 1 voltage |  |
| cell2 | float | volts | Cell 2 voltage |  |
| cell3 | float | volts | Cell 3 voltage | Only 3 cells emitted. |
| ichg | float | amps | Charger current (from INA226) |  |

Cell count note: STM32 emits exactly 3 cells (cell1, cell2, cell3). Flask contains scaffolding for cell4 aliases (app.py:1406-1415), but since cell4 is never emitted, this code path is a silent no-op (harmless legacy). Flask does not error or default; the alias simply remains unpopulated.

### WELD_DONE — Weld Completion Event

Producer: STM32
Consumer: Flask
Relay: ESP32-P4 (raw passthrough)
Frequency: Once per weld completion

Emitted as EVENT,WELD_DONE,key=value,...

| Field | Type | Units | Description | Notes |
| --- | --- | --- | --- | --- |
| peak_current | float | amps | Peak current during weld |  |
| peak_voltage | float | volts | Peak voltage during weld |  |
| peak_power | float | watts | Peak instantaneous power |  |
| avg_power | float | watts | Average power during pulse |  |
| energy_weld_j | float | joules | Integrated weld energy (joule-integration method) | Primary field (see legacy note below). |
| energy_j | float | joules | Legacy/deprecated alias of energy_weld_j | Redundant; both populated from energy_weld_joules. |
| energy_cap_j | float | joules | Energy via cap-bank ΔV method: 0.5 * C * (V_pre² - V_post²) | Separate calculation method. |
| duration_us | uint32 | microseconds | Actual weld pulse duration |  |
| samples | uint16 | — | Number of waveform samples captured |  |

Legacy note: energy_j and energy_weld_j are redundant; both are populated from the same energy_weld_joules variable (main.c:3189-3190). A stale comment at main.c:3157 suggests energy_j was once the cap-bank ΔV method, but that value now resides in energy_cap_j. Flask prefers energy_weld_j and uses energy_j as a fallback (app.py:2067-2070).

### WAVEFORM_* — High-Speed Weld Capture Data

Producer: STM32 (sole producer)
Consumer: Flask
Relay: ESP32-P4 (transparent raw relay — does not parse, consume, or store waveform data)
Frequency: Burst after weld completion

The waveform data is sent as a chunked stream:

#### WAVEFORM_START

Signals the beginning of a waveform burst.

Format: WAVEFORM_START,samples=N,preheat=I1,gap=I2,main=I3

| Field | Type | Units | Description | Notes |
| --- | --- | --- | --- | --- |
| samples | uint16 | — | Total sample count |  |
| preheat | uint16 | sample index | Sample index where preheat phase ends | Zero if no preheat. |
| gap | uint16 | sample index | Sample index where gap phase ends | Zero if no gap. |
| main | uint16 | sample index | Sample index where main weld phase ends | Typically equals samples. |

Important: Phase boundaries in WAVEFORM_START are sample indices (0-based array positions), not time offsets. Contrast with WAVEFORM_PHASES below.

#### WAVEFORM_DATA

Chunked sample data.

Format: WAVEFORM_DATA,timestamp_us,voltage_volts,current_amps,timestamp_us,voltage_volts,current_amps,...

- Positional CSV: triplets of (timestamp_us, voltage_volts, current_amps).

- timestamp_us: Microseconds since weld pulse start.

- Sent in chunks to respect UART/TCP line-length limits.

#### WAVEFORM_END

Signals the end of the waveform burst.

Format: WAVEFORM_END

#### WAVEFORM_PHASES

Phase timing markers (sent after WAVEFORM_END).

Format: WAVEFORM_PHASES,preheat_start=T1,preheat_end=T2,gap_start=T3,gap_end=T4,main_start=T5,main_end=T6

| Field | Type | Units | Description |
| --- | --- | --- | --- |
| preheat_start | uint32 | microseconds | Preheat phase start offset |
| preheat_end | uint32 | microseconds | Preheat phase end offset |
| gap_start | uint32 | microseconds | Gap phase start offset |
| gap_end | uint32 | microseconds | Gap phase end offset |
| main_start | uint32 | microseconds | Main weld phase start offset |
| main_end | uint32 | microseconds | Main weld phase end offset |

Important: Phase boundaries in WAVEFORM_PHASES are microsecond time offsets relative to the weld capture start, not sample indices. This is in contrast to WAVEFORM_START boundaries.

ESP32-P4 handling: The P4 allocates an 8 KB buffer (BUF_SIZE = 8192, welder_main.cpp:57) to accommodate large WAVEFORM_DATA lines and forwards all WAVEFORM_* packets raw to the Flask TCP client via wifi_bridge_broadcast() without parsing or storing the samples locally (welder_main.cpp:358-361).

Legacy single-line format: The original WAVEFORM,timestamp,voltage,current,... (all samples in one line) is now dead code in firmware. Flask retains a _parse_waveform fallback handler (app.py:1229-1237), but it is ignored if chunked waveform assembly is active.

### DISPLAY — Smoothed Voltages for UI Labels

Producer: ESP32-P4 (current firmware) and ESP32_8048S043C (legacy firmware)
Consumer: Flask
Frequency: 1 Hz

Format: DISPLAY,vpack=V1,vcap=V2,cell1=V3,cell2=V4,cell3=V5

| Field | Type | Units | Description |
| --- | --- | --- | --- |
| vpack | float | volts | Smoothed pack voltage |
| vcap | float | volts | Smoothed capacitor voltage |
| cell1 | float | volts | Smoothed cell 1 voltage |
| cell2 | float | volts | Smoothed cell 2 voltage |
| cell3 | float | volts | Smoothed cell 3 voltage |

Purpose: Provides low-frequency smoothed/averaged voltages specifically for dashboard UI labels, reducing update noise compared to the high-frequency STATUS and STATUS2 packets.

Current status: Actively used. ESP32-P4 emits DISPLAY at 1 Hz after receiving STATUS2 data (welder_main.cpp:440-455).

### CELLS — Cell Voltage Request/Response (Legacy)

Producer: ESP32_8048S043C only (legacy firmware)
Consumer: Flask (legacy handler)
Frequency: On-demand (in response to CELLS command from Flask)

Format: CELLS,C1=V1,C2=V2,C3=V3

| Field | Type | Units | Description |
| --- | --- | --- | --- |
| C1 | float | volts | Cell 1 voltage |
| C2 | float | volts | Cell 2 voltage |
| C3 | float | volts | Cell 3 voltage |

Legacy status: Only the old-board firmware (ESP32_8048S043C/src/main.cpp:1876-1882) produces this packet when it receives a CELLS command from Flask. The current ESP32-P4 firmware does not produce CELLS packets, and the STM32 ignores the CELLS command. Flask retains the CELLS request and parser for backward compatibility with old hardware (app.py:1037, 1159-1161).

Superseded by: DISPLAY packets (which provide similar smoothed voltage data at 1 Hz) and continuous STATUS2 telemetry.

## Non-Packet Notes

### WAVEFORM_SAMPLE (Not a Protocol Packet)

WAVEFORM_SAMPLE_INTERVAL_US is a firmware timing constant in STM32G474CE/src/main.c:362, defining the ADC sampling period (50 µs = 20 kHz). It is not a protocol packet type and should never be documented as one.

## Design History and Legacy Notes

- Redundant fields: Several fields exist in duplicate for backward compatibility with older Flask versions:

  - STATUS.joule_target is an alias of joule_target_j.

  - WELD_DONE.energy_j is an alias of energy_weld_j.

  - Flask parsers handle both variants; newer code prefers the _j suffixed versions.

- Cell count evolution: The system was designed for 3-cell LiFePO4 packs. Flask scaffolding for cell4 exists but is unused (no runtime error; silent no-op).

- Waveform format migration: The protocol migrated from a single-line WAVEFORM packet (all samples in one line) to the chunked WAVEFORM_START / DATA / END / PHASES stream to handle UART line-length limits at high sample rates. Flask retains the old parser as a fallback, but current firmware does not emit the old format.

- ESP32-P4 as a transparent bridge: The P4 acts as a relay for most STM32 telemetry (STATUS2, WAVEFORM_*, WELD_DONE), forwarding packets raw without interpretation. It enriches only the STATUS packet with WiFi/system/energy fields and produces the DISPLAY packet independently.

## 🔧 Protocol Cleanup Roadmap (3S → future 4S)

Current reality (July 2026): 3-cell pack. STATUS2 emits cell1, cell2, cell3 only.

Future expansion: 4-cell pack in development. cell4 aliases/parsing in Flask (app.py:1406-1415) are intentional forward-compatibility scaffolding and must remain untouched — they will activate when the new 4S hardware emits cell4 in STATUS2.

### Known redundant fields (safe to consolidate in a future coordinated cleanup):

- STATUS.joule_target ← duplicate of joule_target_j

  - Where: STM32 emitter (main.c:679-680) + Flask parser (app.py:843)

  - Issue: Both fields emitted in joule control mode, both parsed by Flask — identical values

  - Recommendation: Keep joule_target_j (explicit units), drop joule_target

  - Scope: 3-file change (STM32 emitter, ESP32-P4 relay if it parses STATUS, Flask parser)

- WELD_DONE.energy_j ← duplicate of energy_weld_j

  - Where: STM32 emitter (main.c:3189-3190) + Flask parser (app.py:2067-2070)

  - Issue: Both fields emitted in WELD_DONE, both parsed by Flask — identical values

  - Stale comment: The in-code comment at main.c:3157 claiming energy_j is the “cap-bank ΔV source-of-truth” is outdated; energy_cap_j now holds that value

  - Recommendation: Keep energy_weld_j (explicit meaning), drop energy_j

  - Scope: 2-file change (STM32 emitter, Flask parser)

### Legacy packet paths (current ESP32-P4 vs old ESP32-S3):

- CELLS packet: Only produced by legacy ESP32_8048S043C firmware on Flask CELLS command

  - Current ESP32-P4 does not emit it; STM32 ignores the command

  - Flask still requests it on connect if REQUEST_CELLS_ON_CONNECT=True (app.py:1037)

  - Status: Deprecated; superseded by DISPLAY + continuous STATUS2 telemetry

- DISPLAY packet: Produced by both legacy ESP32-S3 and current ESP32-P4

  - Flask accepts from either source

  - Status: Active; dual-source emission is intentional for backward compatibility

Timing: These cleanups are non-urgent — all redundant fields work correctly today due to Flask’s tolerant parsing. Coordinate the changes when the 4S hardware/firmware work begins or during a dedicated protocol cleanup pass.

## Breaking Change Policy

Any modification to: - Field names - Field order - Field types - Units - Packet structure

requires coordinated updates to all four codebases: 1. STM32G474CE/src/main.c (emitter) 2. ESP32P4/main/welder_main.cpp and wifi_bridge.cpp (relay + enrichment) 3. ESP32_8048S043C/src/main.cpp (legacy firmware, if still supported) 4. Spot-Welder-Server/app.py (Flask parser)

Recommended approach: Prefer additive changes (append new fields to the end of packets) over reordering or renaming existing fields, to maintain backward compatibility during transitions.

Document revision: Initial version based on firmware audit (July 2026)