# Known Issues

## 1. ✅ RESOLVED: STM32 wireless/SD firmware updates now work reliably (Katapult bootloader)

### Summary

| What works NOW | ESP32-P4 self-update from SD ✅ · STM32 wireless update from ESP32 ✅ · STM32 SD update ✅ |
| --- | --- |
| What USED to fail | STM32 update from SD/WiFi (pre-Katapult, ROM bootloader approach) |
| How it was fixed | Katapult bootloader — a persistent 8 KiB bootloader at 0x08000000 that handles UART firmware updates with CRC validation, no BOOT0/NRST pins required |
| Status | PRODUCTION-READY (commit 29be8a0, 2026-06-21) |
| Details | See docs/firmware/KATAPULT_BOOTLOADER_SETUP.md |

### What Katapult is and how it fixed the problem

Katapult (formerly CanBoot) is a persistent bootloader that sits in the first 8 KiB of flash (0x08000000) and runs on every boot. It checks for a software update request signature in RAM; if present, it stays in update mode and speaks a simple, CRC-checked protocol over USART1 @ 250000 baud 8N1 — no parity, no BOOT0 pin required. If no request, it immediately jumps to the main app at 0x08002000.

Why this solved everything: 1. No BOOT0/NRST pins needed — the app writes a magic signature to a known RAM address and warm-resets; Katapult sees the signature and stays in bootloader mode. 2. No parity complexity — Katapult uses 8N1 (same as the app), not the ROM’s 8E1. 3. CRC validation — every block is CRC-checked; a corrupted flash is detected and rejected, not silently written. 4. Un-brickable — even a failed flash leaves Katapult intact; re-run the flash to recover. ST-LINK is now only needed for initial Katapult installation.

Flash methods now working: - Wireless via ESP32-P4: upload .bin to http://<esp-ip>/stm32 or run python3 tools/stm32_flash_wifi.py - SD card: copy firmware to SD, tap “Update STM32” on touchscreen - USB-to-serial from PC: python3 tools/katapult_flash_usb.py /dev/ttyUSB0 firmware.bin

All three paths use the same Katapult protocol and work reliably in production.

### What problems this document USED to describe (historical context)

Before Katapult (commits before 29be8a0, June 2026), wireless STM32 flashing relied on the STM32 factory ROM bootloader (AN3155 protocol). That approach had insurmountable problems:

- BOOT0 pin not wired — the ESP32-8048S043C (legacy board) had no free GPIO to drive STM32 BOOT0, required for ROM bootloader entry.

- Software-only jump failed — the STM32G4 ROM bootloader would not respond after a warm jump from the app (silicon limitation).

- 8E1 parity complexity — ROM bootloader required even parity; switching mid-stream caused reliability issues.

Those problems led to the original workaround: “use an ST-LINK to flash the STM32.”

The ROM bootloader approach is documented in docs/archive/WIRELESS_STM32_FLASH_EXPLAINED.md (kept for historical reference). The Katapult approach replaced it wholesale.

### Current recommendation

For STM32 firmware updates: use wireless/SD flashing via Katapult. See docs/firmware/KATAPULT_BOOTLOADER_SETUP.md for setup and usage.

ST-LINK is still useful for: - Initial Katapult installation (one-time, flash katapult-g474-usart1-pa9pa10.bin to 0x08000000) - Deep debugging (breakpoints, memory inspection) - Emergency recovery if Katapult itself somehow gets corrupted (rare)

But ST-LINK is no longer required for routine firmware updates in the field.

## Other known issues

(No other major issues currently documented. This section will be populated as new issues are discovered.)