# ⚠️ ARCHIVED — SUPERSEDED BY KATAPULT BOOTLOADER

This document is kept for historical reference only.

This was a feasibility study written before implementation. STM32 wireless/SD flashing has since been implemented via the Katapult bootloader (commit 29be8a0, 2026-06-21), which uses a different approach than the ROM bootloader path analyzed here.

For current STM32 wireless/SD flashing: see docs/firmware/KATAPULT_BOOTLOADER_SETUP.md

# Feasibility Report: SD-Card Flashing & STM32 Remote Programming

Goal: Make the spot welder self-contained for firmware updates (no PC): 1. Flash ESP32 firmware from the on-board microSD slot. 2. Flash STM32G474CE firmware via the ESP32 (from WiFi upload or microSD).

Scope: Analysis only — nothing implemented. Findings are based on the current firmware, board definition, the project pin-assignment doc, and ST application notes (AN2606 / AN3155).

## 1. Current SD-Card Support

| Item | Finding |
| --- | --- |
| SD library in platformio.ini | None. lib_deps only has rzeldent/esp32_smartdisplay. No SD, SD_MMC, SdFat, or FS usage. |
| SD code in main.cpp | None. No SD.begin(), no SPI bus for the card, no file I/O. |
| Hardware slot present? | YES. boards/esp32-8048S043C.json defines BOARD_HAS_TF and the pins below. |

### SD-card pins (from board definition)

TF_CS       = GPIO10
TF_SPI_MOSI = GPIO11
TF_SPI_SCLK = GPIO12
TF_SPI_MISO = GPIO13

- The microSD is on a dedicated SPI bus (4 pins, GPIO10–13).

- No conflict with the display (RGB parallel), touch (I2C GPIO19/20), or the STM32 UART (GPIO17/18). These four pins are otherwise unused in the firmware.

- Conclusion: the SD card is fully accessible. It just needs the standard Arduino SD/SPI libraries initialised on a dedicated SPIClass instance.

## 2. ESP32 ↔ STM32 Hardware Connections

### UART link (confirmed, already in use)

| Signal | ESP32 | STM32 | Notes |
| --- | --- | --- | --- |
| ESP RX ← STM32 TX | GPIO17 | PA9 (USART1_TX) |  |
| ESP TX → STM32 RX | GPIO18 | PA10 (USART1_RX) |  |
| Application config | — | — | 2,000,000 baud, 8N1 (both sides) |

Source: main.cpp (STM32_TO_ESP32_PIN 17, ESP32_TO_STM32_PIN 18, STM32Serial.begin(2000000, SERIAL_8N1, ...)) and docs/PIN_ASSIGNMENTS.md (PA9/PA10 = USART1 to ESP32).

### BOOT0 control pin

- NOT wired to the ESP32. BOOT0 does not appear anywhere in the ESP32 code, and the STM32 pin table (docs/PIN_ASSIGNMENTS.md) does not route BOOT0 to the inter-board connector. On the WeAct STM32G474 module BOOT0 is on its own button/pad, not under ESP32 control.

### RESET (NRST) control pin

- NOT wired to the ESP32. NRST is listed as “Hardware reset / Input / Infrastructure” (WeAct reset button + RC), but it is not connected to an ESP32 GPIO.

### Is there any existing reset/boot/jump mechanism?

- ESP32 side: none (no GPIO toggling of STM32 boot/reset).

- STM32 side: no jump-to-system-bootloader routine in main.c (the only 0x1FFF… reference is a factory-calibration read, unrelated).

## 3. STM32G474CE Bootloader Protocol (AN2606 / AN3155)

The STM32G474CE has a built-in ROM system bootloader — no code to write on the STM32 to receive an update; we only need a host that speaks the protocol.

### Bootloader UART (AN2606, “STM32G47xxx/48xxx” section, p.238)

- USART1 on PA9 (TX) / PA10 (RX) is a supported bootloader peripheral.

- This is the exact link the ESP32 already has.

- Bootloader UART format: 8 data bits, EVEN parity, 1 stop bit (8E1) with auto-baud (host sends 0x7F so the bootloader locks onto the baud rate).

- ⚠️ The running app uses 8N1; for bootloader mode the ESP32 must reopen the port as 8E1 (e.g. 115200 8E1). One-line STM32Serial.begin() change.

### Protocol commands needed (AN3155)

| Step | Bytes | Purpose |
| --- | --- | --- |
| Sync / auto-baud | 0x7F → expect ACK 0x79 | Establish connection |
| Get | 0x00 0xFF | Bootloader version + supported commands |
| Get-ID | 0x02 0xFD | Confirm chip = G474 |
| Erase | Extended Erase 0x44 0xBB (mass or page list) | Clear flash |
| Write Memory | 0x31 0xCE + addr+chk + 1–256 byte chunks | Program (start 0x08000000) |
| Read Memory (optional) | 0x11 0xEE | Read-back verify |
| Go | 0x21 0xDE + addr | Jump to new app |
| Framing | every cmd = byte + complement; each block checksummed; each step ACK/NACK | Reliability |
| Note | Read-protection (RDP) must be off, else erase/write are blocked |  |

There is no off-the-shelf “stm32flash for ESP32-Arduino” library, but the protocol is small and well-documented; a focused C++ class (~300–500 lines) is the normal approach (mirrors the open-source stm32flash tool / Arduino STM32duino examples).

## 4. Feasibility Summary

### 4A. ESP32 firmware from microSD — ✅ HIGHLY FEASIBLE

- Slot + dedicated SPI pins exist and are free.

- ESP-IDF/Arduino has first-class support: Update.h can flash an OTA image read from a file (SD.open("/firmware.bin") → Update.writeStream()), then ESP.restart(). This is the standard “SD-card OTA” pattern.

- Reuses the OTA partition scheme already in place (16 MB flash, default partitions). The OTA color-loop display blanking work already done applies here too (blank panel while writing flash).

### 4B. STM32 firmware via ESP32 — ⚠️ FEASIBLE WITH A CAVEAT

The data path (UART) and the target (ROM bootloader on PA9/PA10) are ready. The open question is how to put the STM32 into bootloader mode:

- Option 1 — Hardware BOOT0 + NRST control (robust / recommended). Wire 2 ESP32 GPIOs → STM32 BOOT0 and NRST. ESP32 drives BOOT0 high, pulses NRST → STM32 enters system bootloader every time, even if the app is bricked. Requires a small hardware modification (2 wires + the ESP32 must expose 2 free GPIOs — see §5).

- Option 2 — Software “jump to bootloader” (no hardware mod). Add a command to the STM32 app (e.g. ENTER_BOOTLOADER) that de-inits peripherals and jumps to system memory 0x1FFF0000. ESP32 sends the command, then talks AN3155. No wiring change. Caveat: only works while the STM32 app is healthy enough to receive the command — a failed/corrupt flash can leave no recovery path without BOOT0/NRST.

- Best practice: do both — Option 2 for convenience, Option 1 as the un-brickable fallback.

## 5. ESP32 Free-GPIO Reality Check (for Option 1)

Pins already consumed: RGB panel (8,3,46,9,1,5,6,7,15,16,4,45,48,47,21,14, 39,40,41,42), backlight (2), touch (19,20,38), SD (10,11,12,13), STM32 UART (17,18), button (0). The ESP32-S3 OPI PSRAM (qio_opi) also consumes GPIO35–37, and GPIO43/44 are the USB-serial debug UART.

➡️ Free GPIOs are scarce. Driving BOOT0 + NRST needs 2 spare pins brought out on the board’s expansion header (the Sunton 8048S043C breaks out a handful on its IO/speaker connectors). This must be verified against the physical connector pinout before committing to Option 1.

## 6. Libraries / Code Needed

| Feature | Needed |
| --- | --- |
| SD access | SD.h + SPI.h (bundled with Arduino-ESP32) on a dedicated SPIClass, pins 10–13. |
| ESP32 self-flash from SD | Update.h (bundled). Read .bin → Update.writeStream() → restart. |
| STM32 flashing | New custom Stm32Bootloader class implementing AN3155 (sync, Get-ID, erase, write, go) over STM32Serial reconfigured to 8E1. |
| STM32 image source | Either a .bin on SD, or received over WiFi (HTTP upload / the existing TCP bridge) and buffered to SD/PSRAM. |
| STM32 app change (Option 2 only) | Small jump_to_system_bootloader() + a UART command handler. |

## 7. Complexity & Risk

| Item | Complexity | Risk |
| --- | --- | --- |
| ESP32 SD flashing | Low–Med | Low. Dual OTA partitions mean a bad image rolls back; bricking is unlikely. |
| STM32 flashing — protocol | Medium | Medium. AN3155 is simple but needs careful checksums, 8E1 switch, timeouts, retries, verify-after-write. |
| STM32 flashing — boot entry (Opt 1) | Low (code) + HW mod | Brick risk if no fallback. With BOOT0/NRST you always recover. |
| STM32 flashing — boot entry (Opt 2) | Low–Med | Higher brick risk — interrupted flash with no BOOT0/NRST = dead board until SWD/PC rescue. |
| EMI during STM32 flash | — | Welder must be disarmed/idle; verify after write; never flash while charged. |

## 8. Recommended Implementation Approach (phased)

- Phase 1 — ESP32 SD flashing (quick win, low risk). Add SD init on GPIO10–13; “Flash from SD” item on the Setup tab; read /firmware.bin via Update.writeStream(); blank panel during write; restart. Ships value immediately, no STM32 risk.

- Phase 2 — STM32 flashing over the existing UART (Option 2, software jump).

  - Add Stm32Bootloader (AN3155) on the ESP32; reopen UART at 8E1.

  - Add ENTER_BOOTLOADER command + jump_to_system_bootloader() to the STM32 app.

  - Source the STM32 .bin from SD first (simplest), then add WiFi upload.

  - Always read-back verify before Go.

- Phase 3 — Hardware BOOT0/NRST fallback (Option 1, un-brickable). Confirm 2 free header GPIOs, add the 2 wires, implement hardware boot-entry as the recovery path. Strongly recommended before relying on field STM32 updates.

## 9. Key Confirmed Facts (evidence)

- SD slot present, pins free — boards/esp32-8048S043C.json (BOARD_HAS_TF, TF_CS=10/MOSI=11/SCLK=12/MISO=13).

- No SD code today — platformio.ini, src/main.cpp (no SD/FS refs).

- STM32 UART = PA9/PA10 ↔ ESP32 GPIO17/18 @ 2 Mbaud 8N1 — main.cpp, docs/PIN_ASSIGNMENTS.md, STM32G474CE/src/main.c (MX_USART1_UART_Init).

- STM32G47x bootloader on USART1 PA9/PA10 @ 8E1 — AN2606 §47.1 (p.238).

- BOOT0 / NRST not wired to ESP32; no existing jump-to-bootloader — code search of both projects.