# Documentation Staleness Audit

Read-only audit — no doc changes made yet. Cross-references git commit history against current source to flag stale/superseded documentation.

Key event that dates the docs: a Katapult bootloader was ported to the STM32G474 and made STM32 wireless/SD flashing “rock solid.” Several docs were written before Katapult and describe the old (failed) ROM-bootloader reality.

## The timeline that matters (git commit dates)

| Date | Commit | Event |
| --- | --- | --- |
| 6/15 | 0c0c279 | KNOWN_ISSUES.md written — “SD flash impossible, use ST-LINK” |
| 6/17 | c79db17 | SD_AND_STM32_FLASHING_FEASIBILITY.md — “analysis only, nothing implemented” |
| 6/20 | b65d428 | WIRELESS_STM32_FLASH_EXPLAINED.md — ROM/BOOT0 software-entry saga |
| 6/20 | b056943+ | Katapult Phase 1 & 2 ported + implemented |
| 6/21 | 29be8a0 | “Fix all Katapult protocol bugs causing STM32 bricking” → Katapult working |
| 6/21 | 48e3164 | UART moved to UART3-IN header (GPIO 27/28) @ 576 kbaud |
| 6/22 | 3311f60 | SD-card firmware flashing added for ESP32-P4 + STM32 |

Proven current reality (from source, not docs): - App relocated: STM32G474CETX_FLASH.ld → FLASH ORIGIN = 0x8002000 (8 KiB Katapult reserve) - main.c: SCB->VTOR = 0x08002000U; + banner BOOT,FW=KATAPULT-APP-v6-RELOCATED-0x08002000 - ESP32-P4 link: STM_APP_BAUD 576000 on GPIO 27/28 (UART3-IN); KATAPULT_BAUD 250000 - Katapult binaries committed in STM32G474CE/katapult/

## Verdicts

| Doc | Verdict | Why |
| --- | --- | --- |
| docs/issues/KNOWN_ISSUES.md | 🔴 STALE — rewrite or archive | Written 6/15, 6 days before Katapult. Whole premise (SD flash impossible, no BOOT0, ROM won’t work via software, ST-LINK required, option-byte unverified) was solved by Katapult. |
| docs/features/SD_AND_STM32_FLASHING_FEASIBILITY.md | 🟠 ARCHIVE (historical) | Pure feasibility study that says “nothing implemented.” It has since been implemented via Katapult. Still lists old GPIO17/18 wiring. Valuable as history, not as current guidance. |
| docs/features/WIRELESS_STM32_FLASH_EXPLAINED.md | 🟠 ARCHIVE (historical) | Documents the abandoned ROM-bootloader / AN3155 / BOOT0+NRST approach (GPIO29–32, 1 Mbaud, ROM @ 0x1FFF0000, 8E1 parity, breadcrumb). Superseded wholesale by Katapult. Great post-mortem, but not how it works now. |
| docs/firmware/KATAPULT_BOOTLOADER_SETUP.md | 🟡 CURRENT — minor update | This is the authoritative, correct doc. Two stale details: says app link is “1,000,000 baud” (now 576000) and Katapult validate step shows -b 250000 on /dev/ttyUSB0 which is fine, but the app-baud/GPIO (27/28) should be noted. |
| README.md | 🟡 UPDATE — stale sections | Lines ~61–110: “STM32 update via ST-LINK,” “SD-Card flashing requires ST-LINK,” “experimental option-byte…unverified.” All pre-Katapult. Should now point to Katapult wireless/SD flashing. |
| docs/hardware/PIN_ASSIGNMENTS.md | 🟢 CURRENT | STM32-side pinout (PA9/PA10, NRST) unchanged by Katapult. The GPIO move was on the ESP side. |
| docs/hardware/BALANCER_SPECIFICATION.md | 🟢 CURRENT | Hardware spec, no flash dependency. |
| docs/hardware/SNUBBER_SPECIFICATION.md | 🟢 CURRENT | Hardware spec. |
| docs/hardware/BUCK_CONVERTER_PIN_ANALYSIS.md | 🟢 CURRENT | Hardware analysis. |
| docs/hardware/FIRMWARE_MEASUREMENT_ANALYSIS.md | 🟢 CURRENT | Measurement analysis. |
| docs/firmware/ESP32_ARCHITECTURE_ANALYSIS.md | 🟢 CURRENT | Architecture; not flash-specific. |
| docs/firmware/JOULE_ALGORITHM_FIX.md | 🟢 CURRENT | Algorithm history. |
| docs/firmware/MODE_SWITCHER_AND_JOULE_CLEANUP.md | 🟢 CURRENT | Feature history. |
| docs/firmware/STATUS_TAB_UI_IMPROVEMENTS.md | 🟢 CURRENT | UI history. |
| docs/firmware/CHANGELOG_P4_UI_STABLE.md | 🟢 CURRENT | Changelog. |

## Proposed actions (for approval — nothing done yet)

- Create docs/archive/ and git mv the two superseded flash docs there:

  - docs/features/WIRELESS_STM32_FLASH_EXPLAINED.md → docs/archive/

  - docs/features/SD_AND_STM32_FLASHING_FEASIBILITY.md → docs/archive/

  - Add a one-line “SUPERSEDED BY KATAPULT_BOOTLOADER_SETUP.md” banner at top of each.

- KNOWN_ISSUES.md — either rewrite the STM32-flash section to reflect Katapult (recommended) or archive it with a superseded note. Your call on which.

- KATAPULT_BOOTLOADER_SETUP.md — fix the app baud (1,000,000 → 576000) and note the ESP link is GPIO 27/28 (UART3-IN).

- README.md — replace the “ST-LINK required / SD unreliable / option-byte unverified” sections with the current Katapult wireless + SD flow.

- Add a note in docs/archive/README.md explaining these are kept for history.