# ESP32-C6 WiFi Daughterboard (external XIAO C6 over SPI)

Status: current production wiring for the ESP32-P4 tier. Why it exists: the CrowPanel’s onboard ESP32-C6 talks to the P4 over SDIO, and that SDIO bus shares the P4’s SDMMC controller with the microSD card. Using the onboard C6 for WiFi therefore blocks the SD card (and vice-versa). The workaround is to leave the onboard C6 unused and add an external Seeed XIAO ESP32-C6 on the P4’s SPI bus (J7 wireless-module header). SPI frees the SDMMC controller, so WiFi and the SD card work at the same time.

## Background — the pin conflict

The Elecrow CrowPanel Advance ESP32-P4 has:

- An onboard ESP32-C6 co-processor for WiFi, wired to the P4 over SDIO (the P4 itself has no native WiFi radio).

- A microSD card slot wired to the P4’s SDMMC peripheral.

The SDIO transport to the onboard C6 and the SD card both need the SDMMC controller / the same signal pins. You can have WiFi or the SD card on that bus, but not both reliably — bringing up esp-hosted over SDIO left the SD card unable to mount (sdmmc_card_init failed).

Because the firmware needs the SD card for firmware-update images (/esp32_firmware.bin, /stm32_firmware.bin) and WiFi for the Flask dashboard / wireless flashing, both are required simultaneously.

## The workaround — external XIAO C6 on SPI

WiFi is provided instead by an external Seeed XIAO ESP32-C6 running the esp-hosted SPI slave firmware, connected to the P4’s J7 wireless-module header over SPI. The onboard C6/SDIO path is no longer used for WiFi, which leaves the SDMMC controller free for the SD card.

The firmware side is unchanged from the app’s point of view: the standard esp_wifi API in wifi_bridge.cpp is transparently forwarded to the XIAO C6 by esp_hosted / esp_wifi_remote (declared in ESP32P4/main/idf_component.yml). Only the transport (SPI vs SDIO) and its pins changed, in ESP32P4/sdkconfig.defaults.

## Wiring (P4 J7 header → XIAO C6)

| Signal | P4 GPIO | XIAO pin | XIAO GPIO | sdkconfig symbol |
| --- | --- | --- | --- | --- |
| MOSI | 48 | D10 | 18 | CONFIG_ESP_HOSTED_SPI_HSPI_GPIO_MOSI |
| MISO | 47 | D9 | 20 | CONFIG_ESP_HOSTED_SPI_HSPI_GPIO_MISO |
| CLK | 26 | D8 | 19 | CONFIG_ESP_HOSTED_SPI_HSPI_GPIO_CLK |
| CS | 29 | D2 | 2 | CONFIG_ESP_HOSTED_SPI_HSPI_GPIO_CS |
| Handshake | 30 | D1 | 1 | CONFIG_ESP_HOSTED_SPI_GPIO_HANDSHAKE |
| Data Ready | 31 | D7 | 17 | CONFIG_ESP_HOSTED_SPI_GPIO_DATA_READY |
| Reset/EN | 32 | RST/EN | — | CONFIG_ESP_HOSTED_SPI_GPIO_RESET_SLAVE (active-LOW) |
| Power | 3V3 | 3V3 | — | — |
| Ground | GND | GND | — | — |

Notes: - Reset pin GPIO32 is the pin freed up when the STM32 BOOT0/NRST self-test was removed (STM32 updates now go through the Katapult bootloader, so the P4 no longer needs to drive STM32 reset/boot pins). Reset is active-low (the XIAO EN pin: LOW = held in reset, HIGH = running). - Do NOT connect XIAO D4/D5 (GPIO22/23) — those are physically tied to the P4’s I²C bus (GT911 touch + backlight controller). - Use short jumpers (< 10 cm) on the SPI lines to keep the 10 MHz clock clean.

## Key config settings (ESP32P4/sdkconfig.defaults)

CONFIG_ESP_HOSTED_ENABLED=y
CONFIG_ESP_HOSTED_SPI_HOST_INTERFACE=y      # SPI transport (SDIO/UART/SPI-HD disabled)
CONFIG_ESP_HOSTED_IDF_SLAVE_TARGET="esp32c6"
CONFIG_ESP_HOSTED_SPI_MODE=3                 # Mode 3 (CPOL=1, CPHA=1) for non-ESP32 slave
CONFIG_ESP_HOSTED_SPI_FREQ_ESP32C6=10        # 10 MHz — do NOT raise
CONFIG_ESP_HOSTED_SPI_RESET_ACTIVE_LOW=y     # XIAO EN is active-low

⚠️ Do not raise the SPI clock above 10 MHz. The XIAO slave pins are routed through the ESP32-C6 GPIO matrix (not the dedicated IOMUX SPI pins), which adds enough input delay that above ~10 MHz the slave can no longer reliably sample MOSI. 20 MHz was tried first and failed for this reason.

## Requirements & behaviour

- The XIAO C6 must be flashed with the matching esp-hosted SPI slave firmware (v2.12.9 range) before WiFi will come up.

- If the XIAO is missing, unflashed, or mis-wired, wifi_stack_init() fails gracefully: WiFi is disabled for that boot and the welder UI keeps running (see wifi_bridge.cpp — the C6 radio bring-up is intentionally non-fatal).

## See also

- ESP32P4/SPI_WIFI_INTEGRATION_CHECKLIST.md — step-by-step wiring / flashing / boot-log verification and troubleshooting.

- docs/hardware/PIN_ASSIGNMENTS.md — STM32-side pin map.