# STM32G474CE Master Pin Assignment Document

Project: Capacitive Discharge Spot Welder
MCU: STM32G474CEU6 — UFQFPN48 package
Core: ARM Cortex-M4F, 170 MHz, FPU, DSP
Flash/RAM: 512 KB / 128 KB
Author: JQ-DHJ
Revision: 1.0 — Final Hardware Configuration
Date: 2026-05-10

## Table of Contents

- Pin Summary

- Weld Control

- Buck Charger

- Balancer (4-Cell)

- Analog Inputs

- Communication

- Digital I/O

- Debug Interface

- System / Power Pins

- Complete Pin Table

- Available Pins for Future Expansion

- Configuration Notes

- Conflicts and Special Considerations

- Cross-Reference

## Pin Summary

| Category | Pins Used | Pin List |
| --- | --- | --- |
| Weld Control | 3 | PA8, PB10, PA6 |
| Buck Charger | 3 | PB9, PB1, PB8 |
| Balancer (4-cell) | 4 | PB3, PB4, PB5, PB6 |
| Analog Inputs | 5 | PA0, PA1, PA2, PA3, PA4 |
| USART1 (ESP32) | 2 | PA9, PA10 |
| I2C1 (INA226 sensors) | 2 | PA15, PB7 |
| Digital I/O | 3 | PB2, PB12, PC6 |
| Debug (SWD) | 2 | PA13, PA14 |
| Total GPIO Used | 24 |  |
| System/Power | 12 | VBAT, VREF+, VDDA, VDD×3, VSS, NRST, OSC×4 |
| Total Pins Used | 36 | of 48 package pins |
| Available for expansion | 12 | See Available Pins |

## Weld Control

Controls two banks of 8× IRL40SC228 MOSFETs (16 total) via two UCC21520ADW isolated half-bridge gate drivers. Each gate driver receives a single PWM/enable signal and drives high-side and low-side outputs to 8 paralleled MOSFETs.

| Pin | Pkg Pin | AF | Peripheral | Net Name | Direction | Description |
| --- | --- | --- | --- | --- | --- | --- |
| PA8 | 30 | AF6 | TIM1_CH1 | WELD_GATE_1 | Output | PWM drive → UCC21520 U18 (MOSFETs Q3–Q10, Groups 1+2) |
| PB10 | 22 | AF6 | TIM1_CH3¹ | WELD_GATE_2 | Output | PWM drive → UCC21520 U16 (MOSFETs Q11–Q18, Groups 3+4) |
| PA6 | 14 | GPIO | — | WELD_FAULT | Input | Fault feedback from weld gate drivers (active low) |

### Electrical Configuration — Weld Control

| Pin | Mode | Pull | Speed | Drive | Notes |
| --- | --- | --- | --- | --- | --- |
| PA8 | AF Push-Pull | No pull | High | — | TIM1_CH1: 10 kHz PWM, 12-bit, preload OFF for immediate CCR update from ISR |
| PB10 | AF Push-Pull | No pull | High | — | TIM1_CH3: mirrors CH1 timing for synchronised dual-bank firing |
| PA6 | Input | Pull-up | — | — | 10 kΩ external pull-up on PCB; low = fault |

¹ Alternate option: PB10 can also map to TIM2_CH3 (AF1) or USART3_TX (AF7). Using TIM1_CH3 (AF6) keeps both weld channels on the same timer for precise synchronisation. If independent timing is needed, TIM2_CH3 is available.

### Gate Driver Details

| Driver | IC | Ref Des | Input Pin | MOSFET Group | Output Rail |
| --- | --- | --- | --- | --- | --- |
| #1 | UCC21520ADW | U18 | INA (pin 1) ← PA8 | Q3–Q10 (8× IRL40SC228) | 10V_GATE via LM7810 |
| #2 | UCC21520ADW | U16 | INA (pin 1) ← PB10 | Q11–Q18 (8× IRL40SC228) | 10V_GATE via LM7810 |

- DIS (pin 5) on both drivers tied to WELD_FAULT with 10 kΩ pull-down to CTRL_GND

- VCCI (pin 3) powered from 5V_CLEAN

- VDDA/VDDB (pins 16/11) powered from 10V_GATE rail

## Buck Charger

Synchronous buck converter charges the supercapacitor bank from the battery pack. Uses UCC21520ADW (U14) as the isolated half-bridge driver for high-side (Q2) and low-side (Q1) IRL40SC228 MOSFETs through an inductor (L1: CSQX2918L-8R2MC, 8.2 µH).

| Pin | Pkg Pin | AF | Peripheral | Net Name | Direction | Description |
| --- | --- | --- | --- | --- | --- | --- |
| PB9 | 47 | AF10² | TIM8_CH3 | CHARGE_GATE_H | Output | High-side gate → UCC21520 U14 INA (pin 1) |
| PB1 | 18 | AF4² | TIM8_CH3N | CHARGE_GATE_L | Output | Low-side gate → UCC21520 U14 INB (pin 2) |
| PB8 | 46 | GPIO | — | CHARGE_FAULT | Input | Fault input from charge buck converter |

### Electrical Configuration — Buck Charger

| Pin | Mode | Pull | Speed | Notes |
| --- | --- | --- | --- | --- |
| PB9 | AF Push-Pull | No pull | High | TIM8_CH3: complementary PWM with dead-time |
| PB1 | AF Push-Pull | No pull | High | TIM8_CH3N: complementary output of PB9 |
| PB8 | Input | Pull-up | — | ⚠️ BOOT0 function — see Special Considerations |

² AF verification: PB9 → TIM8_CH3 is AF10 per STM32G474 datasheet Table 13. PB1 → TIM8_CH3N is AF4. Both confirmed for UFQFPN48 package.

### Buck Converter Specifications

| Parameter | Value | Notes |
| --- | --- | --- |
| Topology | Synchronous buck | High-side + low-side MOSFETs |
| Switching frequency | Configurable via TIM8 | Target: 50–100 kHz |
| Inductor | CSQX2918L-8R2MC (8.2 µH) | Coilcraft, high current rated |
| High-side MOSFET | IRL40SC228 (Q2) | V_DS = 150 V, R_DS(on) = 3.2 mΩ |
| Low-side MOSFET | IRL40SC228 (Q1) | Same as above |
| Gate driver | UCC21520ADW (U14) | Isolated half-bridge driver |
| Gate supply | LM7810 (U17) → 10V_GATE | 10 V regulated gate drive voltage |
| Protection | SMF15CA TVS diodes (D1, D2) | Across each gate–source |

## Balancer (4-Cell)

Passive cell balancer using a TLP291-4 quad optocoupler and IRLML6244TRPBF N-channel MOSFETs. Each channel bleeds ~1.26 A through 3× paralleled 10 Ω resistors (3.33 Ω effective). See BALANCER_SPECIFICATION.md for full circuit details.

| Pin | Pkg Pin | AF | Peripheral | Net Name | Direction | Description |
| --- | --- | --- | --- | --- | --- | --- |
| PB3 | 41 | GPIO | — | BAL_CELL1 | Output | TLP291-4 Ch1 LED → Cell 1 (bottom) |
| PB4 | 42 | GPIO | — | BAL_CELL2 | Output | TLP291-4 Ch2 LED → Cell 2 |
| PB5 | 43 | GPIO | — | BAL_CELL3 | Output | TLP291-4 Ch3 LED → Cell 3 |
| PB6 | 44 | GPIO | — | BAL_CELL4 | Output | TLP291-4 Ch4 LED → Cell 4 (top) |

### Electrical Configuration — Balancer

| Pin | Mode | Pull | Speed | Drive | Notes |
| --- | --- | --- | --- | --- | --- |
| PB3 | Output Push-Pull | No pull | Low | 3.3 V / 470 Ω | JTDO in JTAG mode — safe for SWD-only |
| PB4 | Output Push-Pull | No pull | Low | 3.3 V / 470 Ω | NJTRST in JTAG mode — safe for SWD-only |
| PB5 | Output Push-Pull | No pull | Low | 3.3 V / 470 Ω | No debug conflict |
| PB6 | Output Push-Pull | No pull | Low | 3.3 V / 470 Ω | No conflict (I2C1 moved to PA15/PB7) |

- Batch mask: 0x0078 (bits 3–6) on GPIOB ODR for simultaneous enable/disable

- Default state: All LOW (balancers off) at power-on

- LED forward current: I_F = (3.3 V − 1.2 V) / 470 Ω ≈ 4.5 mA (within TLP291-4 spec)

## Analog Inputs

Five analog channels across ADC1 and ADC2. ADC configuration: 12-bit resolution, PCLK/4 clock, single-ended, right-aligned data.

| Pin | Pkg Pin | Channel | Peripheral | Net Name | Description |
| --- | --- | --- | --- | --- | --- |
| PA0 | 8 | ADC1_IN1 | ADC1 | THERM_IN | NTC thermistor — temperature monitoring |
| PA1 | 9 | ADC1_IN2 | ADC1 | SHUNT_P_ADC | AMC1302 OUTP — current sense positive |
| PA2 | 10 | ADC1_IN3 | ADC1 | SHUNT_N_ADC | AMC1302 OUTN — current sense negative |
| PA3 | 11 | ADC1_IN4 | ADC1 | VCAP_OUTP | AMC1311B OUTP — cap voltage positive leg |
| PA4 | 12 | ADC2_IN17 | ADC2 | VCAP_OUTN | AMC1311B OUTN — cap voltage negative leg |

### Electrical Configuration — Analog Inputs

| Pin | Mode | Pull | Speed | Notes |
| --- | --- | --- | --- | --- |
| PA0 | Analog | No pull | — | 10 kΩ NTC divider; slow-moving signal |
| PA1 | Analog | No pull | — | Differential pair with PA2; fast current capture |
| PA2 | Analog | No pull | — | Differential pair with PA1; fast current capture |
| PA3 | Analog | No pull | — | Differential pair with PA4; cap voltage measurement |
| PA4 | Analog | No pull | — | ⚠️ Only available on ADC2 (IN17), not ADC1 |

### ADC Operating Modes

| Mode | Channels | Trigger | Use Case |
| --- | --- | --- | --- |
| Single-channel | ADC1_IN1 (PA0) | Software | Periodic thermistor reading |
| Scan mode | ADC1_IN2,3,4 (PA1-3) | Software | Fast current + voltage during weld pulse |
| Single-channel | ADC2_IN17 (PA4) | Software | Cap voltage negative leg |

Note: ADC1 switches between single-channel and 3-channel scan mode at runtime. adcPrepareFastCurrentChannels() reconfigures for weld pulse capture; normal polling reverts to single-channel reads.

### Isolated Amplifier Signal Chain

Battery Current → Shunt Resistor → AMC1302 (±50 mV) → OUTP (PA1) / OUTN (PA2)
                                                         ↕ 1.44 V ± 0.25 V differential

Cap Voltage → Divider → AMC1311B (0–2 V) → OUTP (PA3) / OUTN (PA4)
                                               ↕ 1.44 V ± 0.25 V differential

## Communication

### USART1 — ESP32 Display Controller Link

| Pin | Pkg Pin | AF | Peripheral | Net Name | Direction | Description |
| --- | --- | --- | --- | --- | --- | --- |
| PA9 | 31 | AF7 | USART1_TX | USART1_TX | Output | STM32 → ESP32 data/status |
| PA10 | 32 | AF7 | USART1_RX | USART1_RX | Input | ESP32 → STM32 commands |

| Parameter | Value | Notes |
| --- | --- | --- |
| Baud rate | 2,000,000 bps | High speed for real-time data streaming |
| Format | 8N1 | 8 data bits, no parity, 1 stop bit |
| Mode | Interrupt RX | HAL_UART_Receive_IT for incoming commands |
| Pull | No pull | Direct connection, no external pull-ups |
| GPIO speed | Very High | Required for 2 Mbps signaling |

### I2C1 — INA226 Voltage/Power Monitors

| Pin | Pkg Pin | AF | Peripheral | Net Name | Direction | Description |
| --- | --- | --- | --- | --- | --- | --- |
| PA15 | 38 | AF4 | I2C1_SCL | I2C_SCL | Open-drain | Clock line — 3 INA226 sensors |
| PB7 | 45 | AF4 | I2C1_SDA | I2C_SDA | Open-drain | Data line — 3 INA226 sensors |

| Parameter | Value | Notes |
| --- | --- | --- |
| Speed | 100 kHz | Standard mode; adequate for 150 ms poll cycle |
| Pull-ups | 4.7 kΩ | External pull-ups to 3V3_CLEAN (R4, R5) |
| GPIO mode | AF Open-Drain | Required for I2C bus specification |
| Address space | 3 devices | 0x40, 0x41, 0x44 |

### I2C Device Map

| Address | Ref Des | IC | Measurement | Scaling Factor | Calibration Date |
| --- | --- | --- | --- | --- | --- |
| 0x40 | U8 | INA226 | V_PACK (full pack) | 1.001149 | 2025-05-08 |
| 0x41 | U10 | INA226 | V_LOW (node 1) | 1.001327 | 2025-05-08 |
| 0x44 | U12 | INA226 | V_MID (node 2) | 0.986048 | 2025-05-08 |

- INA226 config register: AVG=64, V_BUS CT=1.1 ms, V_SHUNT CT=1.1 ms, continuous mode

- Poll interval: 150 ms (matches 64-sample conversion time: 64 × 1.1 ms × 2 ≈ 141 ms)

- Cell voltages derived: Cell1 = V_LOW, Cell2 = V_MID − V_LOW, Cell3 = V_PACK − V_MID

### ESP32 Display Link (J16 Connector)

| J16 Pin | Signal | Notes |
| --- | --- | --- |
| 1 | 5V_CLEAN | Power supply to ESP32 |
| 2 | CTRL_GND | Ground reference |
| 3 | USART1_RX | ESP32 TX → STM32 RX (PA10) |
| 4 | USART1_TX | STM32 TX → ESP32 RX (PA9) |

## Digital I/O

| Pin | Pkg Pin | AF | Peripheral | Net Name | Dir | Description |
| --- | --- | --- | --- | --- | --- | --- |
| PB12 | 25 | GPIO | — | PEDAL_IN | Input | Foot pedal trigger (active low) |
| PB2 | 19 | GPIO | — | CHARGER_EN | Output | Charger enable control (active high) |
| PC6 | 29 | GPIO | — | STATUS_LED | Output | Status indicator LED |

### Electrical Configuration — Digital I/O

| Pin | Mode | Pull | Speed | Notes |
| --- | --- | --- | --- | --- |
| PB12 | Input | Internal pull-up | — | Active low: pressed = GND, released = VCC |
| PB2 | Output Push-Pull | No pull | Low | HIGH enables charger; safety-interlocked with INA226 health |
| PC6 | Output Push-Pull | No pull | Low | General status LED; blink patterns indicate state |

## Debug Interface

| Pin | Pkg Pin | Function | Notes |
| --- | --- | --- | --- |
| PA13 | 36 | SWDIO | Serial Wire Debug data — reserved, do not reassign |
| PA14 | 37 | SWCLK | Serial Wire Debug clock — reserved, do not reassign |

- SWD-only configuration (not full JTAG) — frees PB3 (JTDO) and PB4 (NJTRST) for GPIO

- Debug connector: standard 2-pin SWD header on control PCB

## System / Power Pins

| Pin Name | Pkg Pin | Function | Notes |
| --- | --- | --- | --- |
| VBAT | 1 | Battery backup domain | Tied to VDD if no backup used |
| PC13 | 2 | RTC / TAMP | Limited 3 mA drive — unused |
| PC14 | 3 | OSC32_IN | 32.768 kHz crystal (if used) |
| PC15 | 4 | OSC32_OUT | 32.768 kHz crystal (if used) |
| PF0 | 5 | OSC_IN (HSE) | 8 MHz crystal / oscillator |
| PF1 | 6 | OSC_OUT (HSE) | 8 MHz crystal / oscillator |
| PG10/NRST | 7 | System reset | Active low with internal pull-up |
| VREF+ | 20 | ADC reference voltage | Tied to VDDA (3.3 V) |
| VDDA | 21 | Analog power supply | 3.3 V, 100 nF + 1 µF decoupling |
| VDD | 23 | Digital power supply #1 | 3.3 V, 100 nF decoupling |
| VDD | 35 | Digital power supply #2 | 3.3 V, 100 nF decoupling |
| VDD | 48 | Digital power supply #3 | 3.3 V, 100 nF decoupling |
| VSS | EP | Ground (exposed pad) | Soldered to ground plane |

## Complete Pin Table

Sorted by UFQFPN48 package pin number.

| Pkg Pin | Port.Pin | Function | AF | Peripheral | Net Name | Dir | Group |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | VBAT | Power | — | — | VBAT | — | System |
| 2 | PC13 | System | — | RTC | — | — | System |
| 3 | PC14 | System | — | OSC32 | OSC32_IN | — | System |
| 4 | PC15 | System | — | OSC32 | OSC32_OUT | — | System |
| 5 | PF0 | System | — | HSE | OSC_IN | — | System |
| 6 | PF1 | System | — | HSE | OSC_OUT | — | System |
| 7 | PG10/NRST | System | — | — | NRST | — | System |
| 8 | PA0 | Analog | — | ADC1_IN1 | THERM_IN | Input | Analog |
| 9 | PA1 | Analog | — | ADC1_IN2 | SHUNT_P_ADC | Input | Analog |
| 10 | PA2 | Analog | — | ADC1_IN3 | SHUNT_N_ADC | Input | Analog |
| 11 | PA3 | Analog | — | ADC1_IN4 | VCAP_OUTP | Input | Analog |
| 12 | PA4 | Analog | — | ADC2_IN17 | VCAP_OUTN | Input | Analog |
| 13 | PA5 | Available | — | — | — | — | Free |
| 14 | PA6 | Digital Input | — | GPIO | WELD_FAULT | Input | Weld |
| 15 | PA7 | Available | — | — | — | — | Free |
| 16 | PC4 | Available | — | — | — | — | Free |
| 17 | PB0 | Available | — | — | — | — | Free |
| 18 | PB1 | AF Push-Pull | AF4 | TIM8_CH3N | CHARGE_GATE_L | Output | Charger |
| 19 | PB2 | Digital Output | — | GPIO | CHARGER_EN | Output | Control |
| 20 | VREF+ | Power | — | — | VREF+ | — | System |
| 21 | VDDA | Power | — | — | VDDA | — | System |
| 22 | PB10 | AF Push-Pull | AF6 | TIM1_CH3 | WELD_GATE_2 | Output | Weld |
| 23 | VDD | Power | — | — | VDD | — | System |
| 24 | PB11 | Available | — | — | — | — | Free |
| 25 | PB12 | Digital Input | — | GPIO | PEDAL_IN | Input | Control |
| 26 | PB13 | Available | — | — | — | — | Free |
| 27 | PB14 | Available | — | — | — | — | Free |
| 28 | PB15 | Available | — | — | — | — | Free |
| 29 | PC6 | Digital Output | — | GPIO | STATUS_LED | Output | Control |
| 30 | PA8 | AF Push-Pull | AF6 | TIM1_CH1 | WELD_GATE_1 | Output | Weld |
| 31 | PA9 | AF Push-Pull | AF7 | USART1_TX | USART1_TX | Output | Comm |
| 32 | PA10 | AF Push-Pull | AF7 | USART1_RX | USART1_RX | Input | Comm |
| 33 | PA11 | Available | — | — | — | — | Free |
| 34 | PA12 | Available | — | — | — | — | Free |
| 35 | VDD | Power | — | — | VDD | — | System |
| 36 | PA13 | Debug | — | SWD | SWDIO | Bidir | Debug |
| 37 | PA14 | Debug | — | SWD | SWCLK | Input | Debug |
| 38 | PA15 | AF Open-Drain | AF4 | I2C1_SCL | I2C_SCL | Bidir | Comm |
| 39 | PC10 | Available | — | — | — | — | Free |
| 40 | PC11 | Available | — | — | — | — | Free |
| 41 | PB3 | Digital Output | — | GPIO | BAL_CELL1 | Output | Balancer |
| 42 | PB4 | Digital Output | — | GPIO | BAL_CELL2 | Output | Balancer |
| 43 | PB5 | Digital Output | — | GPIO | BAL_CELL3 | Output | Balancer |
| 44 | PB6 | Digital Output | — | GPIO | BAL_CELL4 | Output | Balancer |
| 45 | PB7 | AF Open-Drain | AF4 | I2C1_SDA | I2C_SDA | Bidir | Comm |
| 46 | PB8 | Digital Input | — | GPIO | CHARGE_FAULT | Input | Charger |
| 47 | PB9 | AF Push-Pull | AF10 | TIM8_CH3 | CHARGE_GATE_H | Output | Charger |
| 48 | VDD | Power | — | — | VDD | — | System |
| EP | VSS | Power | — | — | GND | — | System |

## Available Pins for Future Expansion

12 GPIO pins remain unassigned and available:

| Pin | Pkg Pin | Noteworthy Alternate Functions | Potential Use Cases |
| --- | --- | --- | --- |
| PA5 | 13 | DAC1_OUT2, SPI1_SCK, TIM2_CH1, ADC2_IN13 | DAC output, SPI bus, extra ADC |
| PA7 | 15 | SPI1_MOSI, TIM1_CH1N, ADC2_IN4 | SPI bus, complementary PWM |
| PA11 | 33 | USB_DM, FDCAN1_RX, TIM1_CH4, TIM4_CH1 | USB, CAN bus, extra timer |
| PA12 | 34 | USB_DP, FDCAN1_TX, TIM1_ETR, TIM4_CH2 | USB, CAN bus |
| PB0 | 17 | TIM1_CH2N, TIM3_CH3, ADC1_IN15 | Extra PWM, extra ADC |
| PB11 | 24 | USART3_RX, I2C2_SDA, TIM2_CH4 | Second UART, second I2C bus |
| PB13 | 26 | SPI2_SCK, USART3_CTS, TIM1_CH1N | SPI bus, UART flow control |
| PB14 | 27 | SPI2_MISO, USART3_RTS, TIM1_CH2N | SPI bus, UART flow control |
| PB15 | 28 | SPI2_MOSI, TIM1_CH3N, TIM15_CH2 | SPI bus, complementary PWM |
| PC4 | 16 | USART1_TX³, ADC2_IN5 | Backup USART, extra ADC |
| PC10 | 39 | USART3_TX, UART4_TX, SPI3_SCK | Second UART, SPI3 bus |
| PC11 | 40 | USART3_RX, UART4_RX, SPI3_MISO | Second UART, SPI3 bus |

### Recommended Expansion Priorities

- USB (PA11 + PA12): Native USB-FS for firmware updates or PC data logging

- CAN Bus (PA11 + PA12): FDCAN1 for automotive/industrial integration

- SPI (PA5 + PA7 + PB13–PB15): External flash, additional sensors, or display

- Second UART (PC10 + PC11): USART3 for additional communication channel

- Extra ADC (PB0, PA5, PC4): Additional analog monitoring channels

## Configuration Notes

### Clock Configuration

| Peripheral | Clock Source | Frequency | Notes |
| --- | --- | --- | --- |
| System | HSE → PLL | 170 MHz | Maximum performance |
| APB1 | HCLK/1 | 170 MHz | TIM2–7, USART2/3, I2C1/2 |
| APB2 | HCLK/1 | 170 MHz | TIM1/8/15–17, USART1, SPI1, ADC |
| ADC12 | PCLK/4 | 42.5 MHz | Shared clock for ADC1 and ADC2 |

### GPIO Speed Selection Rationale

| Speed Setting | Used For | Reason |
| --- | --- | --- |
| Very High | USART1 (PA9/PA10) | 2 Mbps requires fast edge rates |
| High | TIM1/TIM8 PWM outputs | Clean gate drive switching edges |
| Low | Balancer GPIOs, LED, charger EN | Reduces EMI; ms-scale response is fine |

### Pull-Up / Pull-Down Summary

| Pin | Pull Type | Value | Location | Purpose |
| --- | --- | --- | --- | --- |
| PA15 | External pull-up | 4.7 kΩ | R5 to 3V3 | I2C specification requirement |
| PB7 | External pull-up | 4.7 kΩ | R4 to 3V3 | I2C specification requirement |
| PA6 | External pull-up | 10 kΩ | PCB | Fault line default-high |
| PB8 | Internal pull-up | ~40 kΩ | STM32 | CHARGE_FAULT default-high |
| PB10 | External pull-down | 10 kΩ | R3 to GND | Ensure gate OFF at power-up |
| PB12 | Internal pull-up | ~40 kΩ | STM32 | Pedal default-high (not pressed) |

### Power Domain Mapping

19.5V Battery Pack
    │
    ├── LM7810 (U17) → 10V_GATE ─── UCC21520 VDDA/VDDB gate drive rails
    │
    ├── 5V_CLEAN ──┬── UCC21520 VCCI (all 3 drivers)
    │              ├── ESP32 (via J16)
    │              └── INA226 sensors (Vbus power)
    │
    └── 3V3_CLEAN ─┬── STM32G474CE VDD / VDDA
                   ├── I2C pull-ups
                   └── Optocoupler LED anodes (via 470 Ω)

Ground domains:
    CTRL_GND ───── STM32, ESP32, INA226 (low-noise control ground)
    GND_H ──────── High-current MOSFET sources, gate drivers (isolated by dashed line in schematic)

## Conflicts and Special Considerations

### ⚠️ PB8 — BOOT0 Function

PB8 (pin 46) serves as the BOOT0 pin on STM32G4 series. When HIGH during reset, the MCU boots from system memory (DFU bootloader) instead of Flash.

Mitigation: - CHARGE_FAULT is an input with pull-up — defaults HIGH - The UCC21520 fault output is open-drain, pulling LOW on fault - A 10 kΩ pull-down resistor must be added on CHARGE_FAULT to ensure PB8 is LOW during power-on/reset, preventing accidental DFU boot - Alternatively, configure nBOOT0 option bit to force Flash boot regardless of PB8 state

### ⚠️ PB3/PB4 — JTAG Debug Pins

PB3 (JTDO) and PB4 (NJTRST) are JTAG pins by default after reset.

Mitigation: - Firmware must call __HAL_AFIO_REMAP_SWJ_NOJTAG() or configure JTAG pins as GPIO during MX_GPIO_Init() before using PB3/PB4 as balancer outputs - SWD debug on PA13/PA14 remains fully functional

### ⚠️ PA4 — ADC2-Only Channel

PA4 maps to ADC2_IN17 and is not available on ADC1. The AMC1311B OUTN signal on PA4 requires separate ADC2 initialization and cannot be included in ADC1 scan sequences.

Impact: - Fast current/voltage capture requires coordinated reads from both ADC1 (PA1–PA3) and ADC2 (PA4) - Currently handled by separate MX_ADC2_Init() and adcPrepareVcapNChannel()

### ⚠️ I2C1 Pin Selection History

I2C1 was moved from PB8/PB9 (default) to PA15/PB7 to avoid: - PB8 BOOT0 conflict (would cause DFU boot if SDA was high during reset) - PB9 conflict with TIM8_CH3 (now used for CHARGE_GATE_H) - PA15 (AF4) and PB7 (AF4) confirmed working with 4.7 kΩ external pull-ups

### Timer Resource Allocation

| Timer | Channel(s) Used | Purpose | Available Channels |
| --- | --- | --- | --- |
| TIM1 | CH1 (PA8), CH3 (PB10) | Weld gate drive (dual) | CH2, CH4, CH1N–CH3N free |
| TIM8 | CH3 (PB9), CH3N (PB1) | Buck charger complementary PWM | CH1, CH2, CH4 free |
| TIM2 | — | Not used | All channels available |
| TIM3 | — | Not used | All channels available |
| TIM4 | — | Not used | All channels available |

## Cross-Reference

| Document | Relevance |
| --- | --- |
| BALANCER_SPECIFICATION.md | Full 4-cell balancer circuit & firmware |
| GPIO_BALANCER_ANALYSIS.md | GPIO selection rationale for PB3–PB6 |
| STM32G474xE Datasheet (DS12288) | Pin alternate functions, electrical specs |
| STM32G4 Reference Manual (RM0440) | Peripheral register details, timer config |
| IRL40SC228 Datasheet | MOSFET specs for weld & charger FETs |
| UCC21520ADW Datasheet | Gate driver pinout & application circuit |
| INA226 Datasheet | I2C address configuration, registers |
| AMC1302 Datasheet | Current sense amplifier signal levels |
| AMC1311B Datasheet | Voltage sense amplifier signal levels |
| Si827x Datasheet | ISOdriver for high dV/dt immunity |

## Revision History

| Rev | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-05-10 | JQ-DHJ | Initial release — final hardware config |

This document is the authoritative reference for all STM32G474CE pin assignments in the spot welder project. Any firmware or hardware changes must update this document first.